SYSTEM OVERVIEW:

The Flight Computer (FC) serves as the "brain" of the launch vehicle. Its primary objective is to maintain vertical stability during the boost phase using Thrust Vector Control (TVC), record flight data, and execute recovery events.

Core Functions:

- Stabilization: Active TVC using servo-actuated motor gimbaling.

- Data Logging: High-frequency recording of altitude, acceleration, and orientation.

- Event Triggering: Apogee detection and parachute deployment.

- Telemetry: Real-time air-to-ground data transmission for monitoring flight status.

Final Component Selection Rationale

The following components constitute the final BOM.

1.1 Flight Controller

- Component: ESP32 Development Module

- Role: Central processing unit for TVC, deployment, and telemetry.

- Justification: High-speed dual-core processor essential for running complex stabilization algorithms without loop-time lag.

1.2 Sensors (Navigation & Environment)

- LSM6DSO32 (6-DOF Accelerometer & Gyroscope)

  - Role: Primary orientation sensor for TVC.

  - Justification: This is a high-performance industrial-grade sensor, offering superior stability and noise immunity compared to the older MPU6050.

- ICM-20602 or MPU6050

  - Role: Redundancy or filtering verification.

- Altimeter: BMP280 or BMP390

  - Role: Measures altitude to detect apogee (highest point).

  - Justification: Crucial for the recovery system. It triggers the parachute when it detects the rocket is descending (altitude drops for 5 consecutive readings).

- High-G Accelerometer: ADXL345 or ADXL377 (Optional)

  - Role: Detecting liftoff and measuring peak G-force.

  - Justification: Standard IMUs often "clip" (max out) at 16g. A High-G accelerometer (up to 200g) ensures we capture the true force of the motor burn.

1.3 Telemetry & Data Logging

- Telemetry: LoRa SX1278 Module (433MHz)

  - Role: Real-time air-to-ground data transmission.

  - Justification: Superior to HC-12. LoRa offers long-range capabilities (3-5 km) and excellent noise immunity, ensuring we receive data even if the rocket spins or tumbles.

- Data Logging: Micro SD Card Module

  - Role: "Black Box" recording.

  - Justification: Wireless telemetry can drop packets. Onboard logging saves data at a high frequency for post-flight analysis.

1.4 Actuators (Motion Control)

- Servos: 3x Metal Gear Servos (MG90S or DS929)

  - Usage:

    - 2x for TVC: Gimbaling the motor nozzle for stability.

    - 1x for Recovery: Actuating the parachute release door/mechanism.

  - Justification: Metal gears are mandatory. Plastic gears will strip under the vibration and aerodynamic loads of launch.

1.5 Power System

- Battery: LiPo (7.4V 2S or 12V 3S)

  - Role: Main power source.

- Buck Converter:

  - Role: Steps down high LiPo voltage to a stable 5V or 6V.

  - Justification: Servos need high current at 5V-6V. Powering them directly from the ESP32 is impossible (it would fry the board), and powering them directly from a 12V LiPo would destroy the servos.

2. Static Test Equipment (Ground Support)

- Load Cell: 50kg or 100kg

- Amplifier: HX711

- Role: Measuring Peak Thrust, Burn Time, and Total Impulse during the ground fire test. This data generates the Thrust Curve needed for flight simulations.

3. Architecture Update: Transition to ESP32

Decision: The Flight Computer architecture is upgraded from a Dual Arduino Nano setup to a Single ESP32 Development Board.

3.1 Why the Change?

Initially, the design proposed two Arduino Nanos (one for control, one for data) to handle 5V logic compatibility. However, the ESP32 has been selected as the final choice for the following reasons:

- Processing Power: The ESP32 operates at 240 MHz (Dual Core) compared to the Arduino Nano's 16 MHz. This allows for faster PID loop calculations required for stable Thrust Vector Control (TVC) while simultaneously logging data.

- Unified System: A single board reduces internal volume, weight, and failure points (wiring) compared to the "2 Nano architecture"2.

Voltage Logic: Most modern sensors (LSM6DSO32, BMP280, SD Cards) operate natively on 3.3V. Using an ESP32 eliminates the need for level shifters that would be required with a 5V Arduino Nano.

# STATIC TEST:

1. Introduction and Objectives

The Static Fire Test is the critical validation phase of the propulsion system. In this test, the rocket motor is ignited while securely restrained to a ground test stand. The primary objectives are:

- Characterize Propulsion: To measure the Thrust Curve, Peak Thrust, and Total Impulse to verify the motor meets the mission profile.

- Validate Avionics: To test the "Dual Nano" setup under high-vibration conditions without risking flight hardware.

- Verify Mechanical Integrity: To ensure the Thrust Vector Control (TVC) mounts and gimballing system can withstand the force of firing.

2. Theoretical Framework

To interpret the test results, the following parameters are defined:

2.1 Thrust Generation and Curve

Thrust is the reaction force described by Newton’s Third Law. The Thrust Curve is a graphical representation of this force over time.

- X-Axis: Time (t) in seconds.

- Y-Axis: Force (F) in Newtons.

- Key Phases: The curve displays the Ignition Transient (spike), Steady State (burn), and Tail-off (pressure drop).

2.2 Impulse Calculations

- Total Impulse: Represents the total energy capacity of the motor, used to classify the motor (Class A, B, C, etc.). It is calculated by integrating the area under the thrust curve:

- Specific Impulse : Measures fuel efficiency (push produced per unit of fuel mass):

(g0 = 9.81 m/s2)

3. Experimental Setup (Hardware & Electronics)

The setup is divided into the Ground Station (Force Measurement) and the Onboard Avionics (Vibration & TVC Validation).

3.1 Ground Test Stand (Thrust Measurement)

Since the onboard accelerometer cannot measure thrust when the rocket is clamped stationary, an external sensor system is required.

- Mechanical Frame: A rigid wooden or metal sled design that clamps the rocket motor horizontally or vertically, allowing for slight axial sliding against the sensor.

- Sensor (Load Cell): A single-point load cell rated for 150% of expected peak thrust (e.g., for a 100N motor, use a 20kg/200N cell) to prevent sensor saturation.

- Amplifier: An HX711 module is used to amplify the millivolt signal from the load cell for the microcontroller.

- Data Logger: An Arduino UNO (distinct from the flight computer) connected via USB to a laptop running a Serial Plotter.

- Sampling Rate: Minimum 80Hz (recommended 100Hz+) to capture the rapid ignition spike.